# CardioTrace: A real-time event-based web interface for wearable-derived RR interval acquisition

# Matías Castillo-Aguilar1, Diego Mabe-Castro2, Cristian Núñez-Espinosa1

# Abstract

Wearable devices enable high-resolution acquisition of RR intervals, yet research-grade control over real-time data capture, event annotation, and export remains limited. We present CardioTrace, an open-source, web-based interface designed for the designed for real-time acquisition and visualization of wearable-derived RR intervals with structured event annotation. The software allows users to mark experimental phases (e.g., rest, exercise, recovery), inspect signals during acquisition, and export time-stamped data for downstream analysis. By emphasizing transparency, reproducibility, and flexible protocol design, this tool bridges the gap between consumer wearable data streams and research-oriented workflows in autonomic studies.

# Keywords

# (maximum of six)

# Metadata

| Nr | Code metadata description | Metadata |
| --- | --- | --- |
| C1 | Current code version | V1.0.0 |
| C2 | Permanent link to code/repository used for this code version | https://github.com/matcasti/cardiotrace |
| C3 | Permanent link to reproducible capsule | https://codeocean.com/capsule/0270963/tree/v1 |
| C4 | Legal code license | All software and code must be released under one of the pre-approved licenses listed in the Guide for Authors, such as Apache License, GNU General Public License (GPL) or MIT License. Write the name of the license you’ve chosen here. |
| C5 | Code versioning system used | For example: svn, git, mercurial, etc. (put ‘none’ if none used) |
| C6 | Software code languages, tools and services used | HTML, CSS and JavaScript |
| C7 | Compilation requirements, operating environments and dependencies | No compilation or installation required. |
| C8 | If available, link to developer documentation/manual | http://mozart.github.io/documentation/ |
| C9 | Support email for questions | matias.castillo@umag.cl |

# Motivation and significance

Wearable devices capable of streaming high-resolution RR intervals (RRi) are increasingly used in exercise physiology, autonomic research, and human performance studies, among others. However, the scientific integration of wearable-derived data is often constrained by limitations at the data acquisition stage rather than by signal analysis itself. Many commercial applications focus on proprietary metrics and post-hoc summaries, offering limited access to raw data, restricted transparency, and insufficient support for precise experimental event annotation. These constraints reduce reproducibility and complicate the use of wearable data in controlled research protocols.

A recurring methodological challenge in studies involving exercise or behavioral interventions is the accurate temporal alignment between physiological signals and the experimental events. In many workflows, event timing is recorded manually or reconstructed retrospectively, introducing uncertainty. While analysis-oriented tools such as Kubios HRV [1] and open-source libraries including NeuroKit2 and PhysioZoo provide robust post-processing capabilities, they depend on externally acquired and pre-segmented data. Conversely, real-time monitoring platforms such as Elite HRV or Firstbeat operate as closed systems with limited control over annotation and export, leaving a gap between real-time data streams and research-grade acquisition workflows.

The software presented here addressed this gap by providing an open, web-based interface for real-time RRi acquisition combined with structured, time-stamped event annotation. The application runs directly in a standard web browser and does not require local installation, lowering technical barriers and facilitating deployment across operating systems and experimental settings. To improve robustness, acquired data are automatically saved at fixed intervals during active sessions, reducing risk of data loss due to common technical interruptions such as Bluetooth disconnections or device shutdowns.

In the intended experimental setting, a researcher connects a compatible wearable device via a web browser, initiates data acquisition, annotates predefined or custom events in real time, and monitors signal quality throughout the session. Upon completion, the full dataset, including RRi and event markers, or RRi data only can be exported in both txt and csv formats for downstream analysis. By emphasizing accessibility, transparency, and fault-tolerant data handling, this software supports more rigorous and reproducible wearable-based research workflows and complements existing analysis-focused tools.

# Software description

Describe the software. Provide enough detail to help the reader understand its impact.

# Software architecture:

# Give a short overview of the overall software architecture; provide a pictorial overview where possible; for example, an image showing the components. If necessary, provide implementation details.

# Software functionalities:

# Present the major functionalities of the software.

# Sample code snippets analysis (optional).

# Illustrative examples

Provide at least one illustrative example to demonstrate the major functions of your software/code.

OPTIONAL: you may include one explanatory  video or screencast that will appear next to your article, in the right hand side panel. Please upload any video as a single supplementary file with your article. Only one MP4 formatted, with 150MB maximum size, video is possible per article. Recommended video dimensions are 640 x 480 at a maximum of 30 frames / second. Prior to submission please test and validate your .mp4 file at  \url{http://elsevier-apps.sciverse.com/GadgetVideoPodcastPlayerWeb/verification} . This tool will display your video exactly in the same way as it will appear on ScienceDirect.

- Impact

The software enables new lines of inquiry by facilitating experimental designs that rely on precise, real-time alignment between physiological signals and experimental events. In particular, it supports phase-specific and event-driven analyses in exercise, recovery, and behavioral intervention studies, where transitions between conditions are critical and difficult to reconstruct retrospectively. By allowing structured annotation during data acquisition, the software makes it feasible to investigate transient autonomic responses, rapid recovery dynamics, and protocol-dependent signal segmentation using wearable-derived RRi. In the context of existing research questions, the software improves methodological rigor by reducing uncertainty associated with post-hoc event reconstruction and manual synchronization.

Compared with workflows that rely on external logging or proprietary applications, the integrated acquisition and annotation approach enables temporal precision and data traceability. The automatic data persistence mechanism further increases robustness by mitigating data loss caused by common technical interruptions, thereby improving data completeness in wearable-based experiments

The software has also influenced daily research practice by simplifying experimental setup and lowering technical barriers to wearable data acquisition. Its browser-based design eliminates the need for local installation or platform-specific software, enabling rapid deployment across operating systems and experimental environments. This accessibility facilitates use by multidisciplinary teams and supports both laboratory and field-based data collection with minimal technical overhead.

At the time of writing, the software is primarily used in stress-recovery paradigm studies, including longevity and aging health, sports and exercise, and extreme-environment exposure. AQUÍ AGREGARÍA ALGO SOBRE OPEN-SOURCE RELEASE, REPRODUCIBILITY AND TRANSPARENCY

The software is not currently used in commercial settings and has not led to the creation of spin-off companies. However, its modular and web-based architecture allows adaptation for applied contexts such as performance monitoring, rehabilitation research, or digital health prototyping, when reliable real-time data acquisition and annotation are required.

# Conclusions

Acknowledgements

Optional. You can use this section to acknowledge colleagues who don’t qualify as a co-author but helped you in some way.

# References

[1]	M.P. Tarvainen, J.-P. Niskanen, J.A. Lipponen, P.O. Ranta-aho, P.A. Karjalainen, Comput. Methods Programs Biomed. 113 (2014) 210–220.