CardioTrace: A real-time event-based web interface for wearable-derived RR interval acquisition

Matías Castillo-Aguilar1,2, Diego Mabe-Castro1, Cristian Núñez-Espinosa1,2 *

1 Centro Asistencial Docente y de Investigación (CADI-UMAG), Punta Arenas, Chile.

2 Escuela de Medicina, Universidad de Magallanes (UMAG), Punta Arenas, Chile.

### *Correspondence

Cristian Núñez-Espinosa, Escuela de Medicina, Universidad de Magallanes, Punta Arenas, Chile. Centro Asistencial de Docencia e Investigación CADI-UMAG, Chile. e-mail: cristian.nunez@umag.cl. Address: Avenida Bulnes 01855, Box 113-D. Phone: +56 61 2201411.

Abstract

Wearable sensors provide accessible, high-resolution RR-interval data for heart rate variability research. Still, research-grade acquisition remains limited by proprietary applications, restricted access to raw data, insufficient event annotation, and weak integration with reproducible analysis workflows. We present CardioTrace, an open-source, browser-based application for real-time acquisition, visualization, event annotation, and export of wearable-derived RR intervals, with specific compatibility with the Polar H10 chest strap via Bluetooth Low Energy. CardioTrace enables researchers to record RR intervals in real time, annotate experimental phases such as rest, exercise, recovery, or custom events, monitor signal behavior during acquisition, and export time-stamped datasets in formats compatible with downstream HRV analysis tools. The application runs entirely client-side in a standard web browser, requires no local installation, stores sessions locally through IndexedDB, and automatically saves data during recording to reduce the risk of data loss. By integrating real-time physiological monitoring, structured event marking, transparent data export, and open-source implementation, CardioTrace addresses a practical methodological gap between consumer wearable data streams and research-oriented autonomic workflows. This software may support reproducible experimental protocols in exercise physiology, behavioral interventions, recovery dynamics, aging research, and field-based physiological studies.

Keywords: heart rate variability; RR intervals; wearable devices; Web Bluetooth API; real-time acquisition; autonomic nervous system; event annotation; open-source software.

Metadata

| Nr | Code metadata description | Metadata |
| --- | --- | --- |
| C1 | Current code version | V1.0.0 |
| C2 | Permanent link to the code/repository used for this code version | https://github.com/matcasti/cardiotrace |
| C3 | Permanent link to reproducible capsule | https://matcasti.github.io/CardioTrace/ |
| C4 | Legal code license | MIT License. |
| C5 | Code versioning system used | git |
| C6 | Software code languages, tools, and services used | HTML, CSS, and JavaScript |
| C7 | Compilation requirements, operating environments, and dependencies | No compilation or installation required. |
| C8 | If available, link to developer documentation/manual | https://github.com/matcasti/cardiotrace |
| C9 | Support email for questions | matias.castillo@umag.cl |

# 1. Motivation and significance

Wearable devices capable of streaming high-resolution RR intervals (RRi) are increasingly used in exercise physiology, autonomic research, and human performance studies, among others [1–3]. However, the scientific integration of wearable-derived data is often constrained by limitations at the data-acquisition stage rather than by signal analysis. Many commercial applications focus on proprietary metrics and post-hoc summaries, offering limited access to raw data, restricted transparency, and insufficient support for precise experimental event annotation. This creates a methodological bottleneck at the acquisition stage: even when robust HRV analysis pipelines are available, the scientific quality of the resulting analysis depends on whether RR intervals were captured, annotated, synchronized, and exported in a transparent and reproducible manner.

A recurring methodological challenge in studies involving exercise or behavioral interventions is accurately aligning physiological signals with experimental events in time. In many workflows, event timing is recorded manually or reconstructed retrospectively, introducing uncertainty. While analysis-oriented tools such as Kubios HRV [4] and open-source libraries including NeuroKit2 [5] and PhysioZoo [6] provide robust post-processing capabilities, they depend on externally acquired and pre-segmented data. Conversely, real-time monitoring platforms such as Elite HRV or Firstbeat operate as closed systems with limited control over annotation and export, leaving a gap between real-time data streams and research-grade acquisition workflows.

Therefore, the novelty of CardioTrace does not lie in replacing established HRV analysis platforms, but in standardizing the acquisition layer that precedes them. By combining real-time RR interval streaming, protocol-specific event annotation, automatic local persistence, and transparent export formats, CardioTrace strengthens the methodological chain between wearable sensors and downstream autonomic analysis.

The software presented here addressed this gap by providing an open, web-based interface for real-time RRi acquisition combined with structured, time-stamped event annotation. The application runs directly in a standard web browser and does not require local installation, lowering technical barriers and facilitating deployment across operating systems and experimental settings. To improve robustness, acquired data are automatically saved at fixed intervals during active sessions, reducing the risk of data loss from common technical interruptions, such as Bluetooth disconnections or device shutdowns.

In the intended experimental setting, a researcher connects a compatible wearable device via a web browser, initiates data acquisition, annotates predefined or custom events in real time, and monitors signal quality throughout the session. Upon completion, the full dataset, including RRi and event markers, or RRi data only can be exported in both txt and csv formats for downstream analysis. By emphasizing accessibility, transparency, and fault-tolerant data handling, this software supports more rigorous and reproducible wearable-based research workflows and complements existing analysis-focused tools.

# 2. Software description

## 2.1. Software architecture:

CardioTrace follows a single-tier, client-side architecture that executes entirely within a standards-compliant web browser, with no server-side components or local installation requirements. Because Web Bluetooth support varies across browsers and operating systems, CardioTrace should be used in environments that support Web Bluetooth. The recommended configuration is Google Chrome or another Chromium-based browser on desktop or Android systems, where Bluetooth Low Energy communication with compatible wearable devices is currently more consistently supported. The application is organized into three functional layers (Figure 1).

Figure 1. Functional layers related to the foundational architecture of the CardioTrace application. The application is divided in three layers: a connectivity layer for data acquisition (left panel), a processing layer where statistics and data quality metrics are computed (middle panel), a presentation layer with plots/animations (right panel), and a persistence layer for in-browser data management (bottom panel).

The connectivity layer interfaces with Bluetooth Low Energy (BLE) wearable devices through the Web Bluetooth API, a browser-native interface that enables direct communication with GATT-based peripherals. Upon connection, the application subscribes to two BLE services: the standard Heart Rate GATT service (UUID 0x180D), which streams HR measurements and RR intervals via the Heart Rate Measurement characteristic (0x2A37), and the Polar Measurement Data (PMD) proprietary service (UUID fb005c80), which enables raw ECG streaming on compatible devices (e.g., Polar H10). An initial calibration window of 8 seconds is applied upon connection; data acquired during this period are discarded to ensure baseline signal stability before recording commences.

The processing layer handles all signal operations in real time using plain JavaScript. Artifact rejection is applied to each incoming RR interval by enforcing physiological validity bounds (250–2000 ms, corresponding to heart rates of 30–200 BPM) and a maximum permissible inter-beat difference of 300 ms. Both the unfiltered (raw) and artifact-rejected (clean) streams are stored in parallel, preserving data provenance. Frequency-domain analysis uses a Lomb–Scargle periodogram [7], which accommodates the non-uniform sampling of RR series without prior interpolation, followed by spectral smoothing with a five-point moving-average filter. The Stress Recovery Index (SRI) is implemented as an exploratory composite display intended to summarize short-term changes in autonomic-related metrics during acquisition. It combines RMSSD, LF/HF ratio, and heart rate recovery rate using predefined weights. Because this index has not yet been independently validated as a clinical or diagnostic marker, it should be interpreted as a visualization aid rather than as a standalone physiological outcome [8].

CardioTrace follows a single-tier, client-side architecture that executes entirely within a standards-compliant web browser, with no server-side components or local installation requirements. However, because Web Bluetooth support varies across browsers and operating systems, the application should be used in environments that support Web Bluetooth. For this reason, the recommended configuration is Google Chrome or another Chromium-based browser on desktop or Android systems, where Bluetooth Low Energy communication with compatible wearable devices is currently more consistently supported.

The persistence and presentation layers manage data storage and visualization. Session data, including RR intervals, event markers, tags, and computed statistics, are persisted to the browser’s IndexedDB (schema version 4) through an auto-save routine triggered every 10 seconds during active recording. Visualization is handled by Plotly.js (v2.27.0), which renders five real-time charts: ECG waveform, RR interval series, 1-minute rolling RMSSD, Poincaré plot, and Power Spectral Density. Export utilities rely on jsPDF (v2.5.1) for PDF report generation and JSZip (v3.10.1) for bulk session download as a compressed archive.

## 2.2. Software functionalities:

CardioTrace exposes the following major functionalities, grouped by workflow stage.

Device connection and calibration. The user initiates a BLE scan through the browser’s device picker, which filters for devices advertising the Heart Rate GATT service. Upon pairing, the application enters an 8-second calibration period before data recording begins, ensuring that transient artifacts introduced at connection do not contaminate the session record. Battery level and signal quality are monitored continuously throughout the session.

Real-time data acquisition and artifact filtering. RR intervals are extracted from each incoming HR Measurement notification. Critically, R-wave detection is performed entirely onboard the wearable device by its proprietary firmware; for the Polar H10, this algorithm has been independently validated against electrocardiographic reference standards across a range of physical activity intensities [9–11], rendering additional R-peak detection at the software level unnecessary. The intervals received by the application therefore, represent already-identified inter-beat intervals rather than raw ECG samples. The artifact rejection implemented in CardioTrace constitutes a supplementary quality-control layer, addressing residual transmission errors, isolated ectopic beats, or motion-induced outliers, and operates through two sequential criteria: a physiological bounds check (250–2000 ms, corresponding to 30–200 BPM) followed by an inter-beat difference gate (≤300 ms). Intervals passing both criteria are appended to the cleaned stream; all intervals received after calibration, regardless of validity, are preserved in a parallel raw stream. For compatible devices, raw ECG data are simultaneously streamed via the Polar PMD service.

Event annotation. At any point during a session, the researcher may insert a time-stamped event marker by activating the Mark Event panel. Eight predefined event types are available (Start, Exercise, Rest, Peak, Recovery, Breathe, Note, and Custom), each optionally accompanied by a free-text annotation. Event time-stamps are stored relative to the post-calibration recording onset and are embedded in all export formats.

Session tagging and automatic persistence. Researchers may attach one or more descriptive tags to a session (e.g., a participant identifier or an experimental condition) before or during recording. Session data, including both RR streams, event markers, computed statistics, and tags, is automatically saved to the browser’s IndexedDB every 10 seconds, providing fault tolerance against Bluetooth disconnections or browser interruptions.

Real-time metrics and visualization. Five interactive charts update continuously: (i) ECG signal trace; (ii) RR interval time series with event marker overlays; (iii) rolling RMSSD; (iv) Poincaré plot; and (v) Lomb–Scargle Power Spectral Density [7] with annotated VLF, LF, and HF bands and LF/HF ratio. LF/HF values are displayed as descriptive spectral indicators. They should not be interpreted as direct measures of sympathovagal balance, particularly during non-stationary periods such as exercise onset or recovery [8]. A five-zone heart rate classification scheme is also displayed in real time. The SRI gauge provides an exploratory composite display of short-term autonomic-related changes.

Data export. Upon session completion, data may be exported as: (i) a CSV file containing time-stamped RR intervals and event markers with a structured metadata header; (ii) a plain TXT file containing RR interval values only, suitable for direct import into analysis tools; or (iii) a formatted PDF report summarising session statistics and chart snapshots. Multiple sessions stored in history can be downloaded simultaneously as a ZIP archive.

Session history management. All sessions saved to IndexedDB are accessible through the history panel, which supports full-text search, tag-based filtering, and sorting by date or duration. Sessions can be individually restored to the active interface, re-exported in any supported format, or permanently deleted.

Table 1. Main CardioTrace functionalities and scientific relevance

| Function | Description | Scientific relevance |
| --- | --- | --- |
| Real-time RR acquisition | Streams RR intervals from compatible BLE wearables | Enables continuous monitoring during experimental protocols |
| Event annotation | Marks rest, exercise, recovery, breathing, peak, notes, or custom events | Supports phase-specific HRV segmentation |
| Raw and filtered streams | Stores original and artifact-filtered RR intervals | Preserves data provenance and quality control |
| Local persistence | Auto-saves sessions every 10 seconds in IndexedDB | Reduces data loss during field or laboratory recordings |
| Visualization | Displays RR series, RMSSD, Poincaré plot, PSD, ECG trace | Supports real-time inspection of signal behavior |
| Export | CSV, TXT, PDF, ZIP formats | Facilitates downstream analysis in Kubios, NeuroKit2, or other tools |
| Open-source architecture | HTML, CSS, JavaScript; MIT license | Supports transparency, auditability, and reproducibility |

# 3. Illustrative examples

The following example illustrates a complete data-acquisition workflow for a structured exercise-recovery protocol, a typical use case in autonomic research.

Scenario: A researcher aims to characterize the acute autonomic response to a submaximal cycling bout and subsequent passive recovery, using a Polar H10 chest-strap worn by a healthy participant.

Step 1: Device connection. The researcher navigates to the CardioTrace application in a Web Bluetooth-compatible browser (e.g., Google Chrome ≥56 on desktop or Android), attaches the Polar H10 to the participant, and clicks Connect Device. The browser device picker displays available BLE peripherals; the researcher selects the Polar H10. The application negotiates both the HR GATT and Polar PMD services, initiates an 8-second calibration period, and then begins recording. Battery level, signal quality, and HR zone are displayed in the connection panel.

Step 2: Baseline recording and event marking. The participant remains seated at rest for five minutes. The researcher observes rising RMSSD values and an SRI score in the Good–Excellent range. At the onset of the rest phase, the researcher opens the Mark Event panel and inserts a Start event with the annotation “baseline rest”. Event markers are visible as vertical lines on the RR interval and rolling RMSSD charts.

Step 3: Exercise phase. The participant begins cycling at 70% of their estimated maximum heart rate. The researcher marks an Exercise event at the transition. In real time, the RR interval chart shows a progressive decrease in inter-beat intervals; the rolling RMSSD declines; and the exploratory SRI display decreases, consistent with the expected reduction in short-term variability during exercise. The Poincaré plot compresses along the SD1 axis, reflecting reduced beat-to-beat variability. The HR zone indicator transitions from Zone 2 (Endurance) to Zone 3–4 (Tempo/Threshold).

Step 4: Recovery phase. At the cessation of exercise, the researcher marks a Recovery event. Over the following minutes, progressive restoration of RR interval variability is evident across all panels. A Rest event is marked once the SRI returns to pre-exercise levels.

Step 5: Session export. The researcher enters a session label in the filename field, adds tags (e.g., participant-01, cycling), and clicks Download. With the CSV format selected and the Include raw data toggle disabled, a file is generated containing the artifact-filtered RR series with embedded event markers, along with a structured metadata header recording the session date, duration, data quality, and event count. The same data may also be exported as a TXT file for direct import into downstream analysis tools such as Kubios HRV [4] or the NeuroKit2 Python library [5]. A PDF report summarising session statistics and chart snapshots is generated via the Report button.

The session is automatically preserved in the history panel under the assigned tags and is retrievable for re-export or review in future browser sessions.

# 4. Technical verification and data integrity

CardioTrace was designed to preserve data integrity during real-time wearable-based acquisition. Technical verification should consider Bluetooth connection stability, correct RR interval streaming, preservation of raw and artifact-filtered data streams, event-marker time-stamp accuracy, automatic session persistence, and consistency of exported CSV and TXT files. Because RR intervals are detected by the wearable firmware and transmitted to the application through Bluetooth Low Energy, CardioTrace should be interpreted as an acquisition, annotation, visualization, and export layer rather than an independent R-peak detection algorithm. Accordingly, validation efforts should focus on data transmission integrity, event-timing accuracy, export consistency, and agreement of exported RR interval files with downstream HRV analysis platforms.

# 5. Impact

CardioTrace may enable new lines of inquiry in wearable-based autonomic research by facilitating experimental designs that require precise, real-time alignment between physiological signals and experimental events. This is particularly relevant for exercise-recovery paradigms, behavioral interventions, aging research, rehabilitation studies, sports science, and field-based assessments in challenging environments, where transitions between conditions are critical and difficult to reconstruct retrospectively. By allowing structured event annotation during acquisition, the software supports phase-specific and event-driven analyses of transient autonomic responses, rapid recovery dynamics, and protocol-dependent HRV segmentation using wearable-derived RR intervals.

Compared with workflows based on external logging, retrospective synchronization, or proprietary applications, CardioTrace integrates acquisition, annotation, visualization, local persistence, and export within a single browser-based environment. This approach improves methodological rigor by reducing uncertainty associated with post-hoc event reconstruction, while automatic session saving mitigates data loss caused by Bluetooth disconnections, browser interruptions, or device-related technical failures. Its installation-free design also lowers technical barriers, enabling rapid deployment by multidisciplinary teams across laboratory and field settings.

The open-source release of CardioTrace under a permissive license further strengthens transparency and reproducibility. Because the application is implemented in standard web technologies without compiled binaries, proprietary runtime dependencies, or external data transmission, researchers can inspect, adapt, and extend the complete acquisition workflow, from BLE communication and calibration logic to artifact-rejection thresholds and export formatting. In this sense, CardioTrace provides a transparent acquisition and annotation layer that complements downstream HRV analysis platforms and may be adapted for applied contexts such as performance monitoring, rehabilitation research, or digital health prototyping.

6. Limitations

Several limitations should be acknowledged. First, CardioTrace depends on the availability and stability of the Web Bluetooth API, which is not uniformly supported across all browsers and operating systems. Second, although the Polar H10 provides validated RR-interval detection, CardioTrace does not independently detect R peaks from raw ECG for its primary RR-interval stream. Therefore, the quality of RR intervals depends partly on the wearable firmware and the Bluetooth transmission process. Third, the implemented artifact rejection rules are intentionally simple and transparent, but they may not replace more advanced correction procedures available in specialized HRV software. Fourth, frequency-domain estimates obtained during real-time acquisition should be interpreted cautiously, especially during short or non-stationary segments such as exercise transitions or early recovery. Finally, the SRI display should be considered exploratory until it is formally validated against established autonomic, recovery, or clinical outcomes.

# 7. Conclusions

# CardioTrace provides an open-source, browser-based solution for real-time RR interval acquisition, visualization, event annotation, local persistence, and export using wearable sensors such as the Polar H10. The application addresses a practical methodological gap in HRV research by improving the temporal alignment between physiological signals and experimental events while maintaining transparency of the acquisition workflow. Rather than replacing specialized HRV analysis software, CardioTrace complements existing platforms by standardizing the acquisition and annotation stage that precedes downstream analysis. Its client-side architecture, lack of installation requirements, automatic local storage, and open-source implementation make it suitable for laboratory, field-based, and intervention studies that require flexible, reproducible RR interval capture. Future work should evaluate event-timing accuracy, cross-platform stability, compatibility with additional BLE devices, and agreement of exported RR interval datasets with established HRV analysis pipelines.

# References

[1] 	M. Castillo-Aguilar, D. Mabe-Castro, D. Medina, C. Núñez-Espinosa, Scientific Reports 15 (2025) 8628.

[2] 	M. Castillo-Aguilar, G. Viscor, L. Sarmiento, J. Sepúlveda, M. Navarrete, C. Núñez-Espinosa, Frontiers in Neuroscience 19 (2026) 1702281.

[3] 	M. Castillo-Aguilar, L.S. Varón, C. Pérez, R. Uribe-Paredes, M.A. Navarrete, C. Nuñez-Espinosa, Aging Cell 25 (2026) e70428.

[4] 	M.P. Tarvainen, J.-P. Niskanen, J.A. Lipponen, P.O. Ranta-Aho, P.A. Karjalainen, Computer Methods and Programs in Biomedicine 113 (2014) 210–220.

[5] 	D. Makowski, T. Pham, Z.J. Lau, J.C. Brammer, F. Lespinasse, H. Pham, C. Schölzel, S.H.A. Chen, Behavior Research Methods 53 (2021) 1689–1696.

[6] 	J.A. Behar, A.A. Rosenberg, I. Weiser-Bitoun, O. Shemla, A. Alexandrovich, E. Konyukhov, Y. Yaniv, Frontiers in Physiology 9 (2018) 1390.

[7] 	P. Laguna, G.B. Moody, R.G. Mark, IEEE Transactions on Biomedical Engineering 45 (1998) 698–715.

[8] 	Task Force of the European Society of Cardiology and the North American Society of Pacing and Electrophysiology, Circulation 93 (1996) 1043–1065.

[9] 	R. Gilgen-Ammann, T. Schweizer, T. Wyss, European Journal of Applied Physiology 119 (2019) 1525–1532.

[10] 	M. Schaffarczyk, B. Rogers, R. Reer, T. Gronwald, Sensors 22 (2022) 6536.

[11] 	V. Chung, L. Chopin, J. Karadayi, J. Grèzes, Sensors 26 (2026) 855.

Conflicts of interests

The authors declare that the research was conducted without any commercial or financial relationships that could be construed as a potential conflict of interest.

Author Contributions

Conceptualization: P.S.-F., M.C.-A., A.S.-L., C.S., and C.N.-E.; Data curation, P.S.-F. and M.C.-A.; Investigation, P.S.-F., M.C.-A., A.S.-L., C.S.-V., C.S. and C.N.-E.; Methodology, SF-P MC-A, C.N.-E.; Supervision, P.S.-F., and C.N.-E.; Formal analysis, P.S.-F., M.C.-A., A.S.-L., C.S.-V., C.S. and C.N.-E.; Visualization, M.C.-A.; Writing–original draft, . P.S.-F., M.C.-A., A.S.-L., C.S.-V., C.S. and C.N.-E.; Writing–review & editing, P.S.-F., M.C.-A., A.S.-L., C.S.-V., C.S. and C.N.-E. All authors have read and agreed to the published version of the manuscript.

Funding

CN-E was funded by ANID Proyecto FONDECYT Regular Nº1250474 and by the Innovation Fund for Competitiveness of the Regional Government of Magallanes and Chilean Antarctica (BIP Code 40042452-0).

Data and code availability

The source code of CardioTrace is publicly available at https://github.com/matcasti/cardiotrace under the MIT License. A live version of the application is available at https://matcasti.github.io/CardioTrace/. No participant-level data are included in this software manuscript.